NAVY TRAINING SYSTEM PLAN

FOR THE

ADVANCED ARRESTING GEAR

PROGRAM

N88-NTSP-A-50-0127/I

JULY 2002

EXECUTIVE SUMMARY

The Advanced Arresting Gear (AAG) will provide the U.S. Navy with the ability to recover all existing and projected carrier based tailhook-equipped air vehicles well into the twenty-first century.  The AAG will be back fit on existing CVN 68 class aircraft carriers and forward fit on CVNX class carriers.  The AAG is responsive to the CVNX MNS (M070-88-96).

The Advanced Arresting Gear (AAG) program will be designed to improve arresting gear structural integrity margins of safety from current MK-7 levels to values needed for future Fleet operational requirements.  In addition, the new AAG will be designed to provide a total life cycle cost savings by reducing both operational and maintenance costs when compared to the MK-7 Arresting Gear.  The AAG will also provide new operational capabilities, including the ability to safely and efficiently recover both heavier and/or faster aircraft and lightweight unmanned air vehicles (goal) that may enter the Fleet in the coming years.

The AAG is currently in the Concept and Technology Development phase of the Defense Acquisition System (DAS) until March 2002.  The DAS System Development and Demonstration starts with a Milestone B decision in May 2002 and will continue until July 2007, followed by a Milestone C decision in August 2007.  Production and Fielding is scheduled from October 2007 and beyond.  Initial Operational Capability must occur when the first ship installed with AAG is deployed in 2016.

The AAG system consists of four units, where a unit is defined as a single recovery wire and associated equipment.  It is envisioned that the AAG deck configuration will utilize a “3 + 1” recovery wire configuration, where a maximum of three recovery wires are rigged on three of the units at any given time.  The remaining unit may be utilized as a spare, enabling a recovery wire to be rigged in the event one of the other units becomes unavailable.

A primary goal of the AAG is to allow recovery operations to be executed using significantly fewer sailors in the arresting gear crew.  The AAG system will have an embedded Health Monitoring (HM) system and an embedded performance monitoring system.  Using Conditional Based Maintenance (CBM) and HM will allow for the monitoring and diagnosis of the Catapults and Arresting Gear.  CBM/HM will instrument critical parameters and use the data obtained to determine the “health” of the systems.  Using these techniques, maintenance requirements can be determined conditionally rather than the current event or time-driven method.

In this early phase of development, no formal training concept has been defined.

TABLE OF CONTENTS

Page

Executive Summary		i

List of Acronyms		iii

Preface		v

PART I  -  TECHNICAL PROGRAM DATA

A.	Nomenclature-Title-Program		I-1

B.	Security Classification		I-1

C.	Manpower, Personnel, and Training Principals		I-1

D.	System Description		I-1

E.	Developmental Test and Operational Test		I-2

F.	Aircraft and/or Equipment/System/Subsystem Replaced		I-2

G.	Description of New Development		I-2

H.	Concepts		I-5

1.	Operational		I-5

2.	Maintenance		I-5

3.	Manning		I-6

4.	Training		I-7

I.	Onboard (In-Service) Training		I-18

J.	Logistics Support		I-19

K.	Schedules		I-20

L.	Government-Furnished Equipment and Contractor-Furnished Equipment

Training Requirements		I-20

M.	Related NTSPs and Other Applicable Documents		I-20

APPENDIX A  -	POINTS OF CONTACT		A-1

| LIST OF ACRONYMS |  |
| --- | --- |
| AAG | Advanced Arresting Gear |
| ABE | Aviation Boatswain’s Mate (Launching and Recovery Equipment) |
| ADMACS | Aviation Data Management And Control System |
| AG | Arresting Gear |
| ALRE | Aircraft Launch Recovery Equipment |
| AZ | Aviation Maintenance Administrationman |
| BIT/BITE | Built-In Test/Built-In Test Equipment |
| CBM | Conditional Based Maintenance |
| CDP | Cross-Deck Pendent |
| CNO | Chief of Naval Operations |
| CROV | Constant Runout (Control) Valve |
| CVN | Aircraft Carrier, Nuclear |
| CVNX | Aircraft Carrier, Nuclear, Experimental |
| DAS | Defense Acquisition System |
| DT | Developmental Test |
| EM | Electrician’s Mate |
| EMALS | ElectroMagnetic Aircraft Launch System |
| FLOLS | Fresnel Lens Optical Landing System |
| FY | Fiscal Year |
| HM | Health Monitoring |
| IAGCS | Integrated Arresting Gear Control System |
| IC | Interior Communications Electrician |
| IETM | Interactive Electronic Technical Manual |
| IFLOLS | Improved Fresnel Lens Optical Landing System |
| IOC | Initial Operational Capability |
| JCTS | Jet Car Test Site |
| MCBOMF | Mean Cycles Between Operational Mission Failures |
| MS | Maintenance Support |
| NA | Not Applicable |
| NAMTRAU | Naval Air Maintenance Training Unit |
| NATTC | Naval Air Technical Training Center |
| NAWCADLKE | Naval Air Warfare Center Aircraft Division Lakehurst |
| NAWCTSD | Naval Air Warfare Center Training System Division |
| NEC | Navy Enlisted Classification |
| NTSP | Navy Training System Plan |
| OPEVAL | Operational Evaluation |
| OPO | OPNAV Principal Official |
| P3I | Pre-Planned Product Improvement |
| PMA | Program Manager, Air |
| QA | Quality Assurance |
| RFT | Ready For Training |
| RALS | Runway Arrested Landing Site |
| SDD | System Development and Demonstration |
| TBD | To Be Determined |
| TD | Training Device |
| TTE | Technical Training Equipment |

PREFACE

This Initial Navy Training System Plan (NTSP) for the Advanced Arresting Gear (AAG) program was developed to update the Initial Advanced Arresting Gear Engine (AAGE) replacement program dated, February 2002.  This Initial NTSP an early look at the Advanced Arresting Gear (AAG) program, and is the second iteration of the Initial NTSP for the AAG program.  This document explores the various employment and support alternatives currently under consideration and includes the program name change from Advanced Arresting Gear Engine replacement program to Advanced Arresting Gear program.  Since it is relatively early in the acquisition process, some definitive data was unavailable for inclusion in this version.  This NTSP is a product of the Training Planning Process Methodology, as outlined in OPNAV Publication P-751-3-9-97.

N88-NTSP-A-50-0127/I

July 2002

PART I - TECHNICAL PROGRAM DATA

A.  NOMENCLATURE-TITLE-PROGRAM

1.  Nomenclature-Title-Acronym.  Advanced Arresting Gear (AAG) Program

2.  Program Element.  0603512N

B.  SECURITY CLASSIFICATION

1.  System Characteristics		Unclassified

2.  Capabilities		Unclassified

3.  Functions		Unclassified

### C.  MANPOWER, PERSONNEL, AND TRAINING PRINCIPALS

OPNAV Principal Official (OPO) Program Sponsor	 CNO (N78)

OPO Resource Sponsor	 CNO (N78)

Functional Mission Sponsor (if applicable)	 CNO (N78)

Developing Agency	 NAVAIRSYSCOM (PMA251)

Training Agency	 CINCLANTFLT (N71)

CINCPACFLT (N70)

CNET (ETS-23)

Training Support Agency	 NAVAIRSYSCOM (PMA205)

Manpower and Personnel Mission Sponsor	 CNO (N12)

NAVPERSCOM (PERS-4, PERS-404)

Director of Naval Training	 CNO (N795)

D.  SYSTEM DESCRIPTION

1.  Operational Uses.  The Advanced Arresting Gear (AAG) will provide the U.S. Navy with the ability to recover all existing and projected, carrier based tailhook-equipped air vehicles well into the twenty-first century.  The AAG will be back fit on existing Aircraft Carrier, Nuclear (CVN) 68 class aircraft carriers and forward fit on Aircraft Carrier, Nuclear, Experimental (CVNX) class carriers.  The AAG is responsive to the CVNX Mission Needs Statement (M070-88-96).

2.  Foreign Military Sales.  No foreign military sales are planned at this time.

E.  DEVELOPMENTAL TEST AND OPERATIONAL TEST.  The Developmental Test (DT) and Evaluation program will be divided into four major divisions, contractor laboratory testing, Jet Car Test Site (JCTS) testing (DT-I, DT-II), Runway Arrested Landing Site (RALS) testing (DT-III), and environmental testing (DT-IV).  Shipboard certification testing and Operational Evaluation (OPEVAL) will be conducted at the completion of developmental testing.

The Concept and Technology Development phase of the Defense Acquisition System (DAS) began in Fiscal Year (FY) 01.  The System Development and Demonstration (SDD) phase will begin in mid-FY03.  Production and Deployment phase is scheduled to begin 3rd quarter FY13.

The SDD system will initially be surface-mounted at the Naval Air Warfare Center Aircraft Division, Lakehurst (NAWCADLKE), New Jersey, the JCTS for testing with deadloads.  Following successful completion of an initial DT program, the system will be moved and installed at the RALS for follow-on DT, Technical Evaluation (TECHEVAL), and OPEVAL.

F.  AIRCRAFT AND/OR EQUIPMENT/SYSTEM/SUBSYSTEM REPLACED.  The AAG replacement program will develop and field a new arresting gear engine to replace the current MK-7 Shipboard Arresting Gear System.  The AAG will be designed to be backfit on existing CVN 68 class aircraft carriers with applications for forward fit on future aircraft carriers.

G.  DESCRIPTION OF NEW DEVELOPMENT

1.  Functional Description.  The new AAG will be designed to improve arresting gear structural integrity margins of safety from current MK-7 levels to values needed for future Fleet operational requirements.  In addition, the new AAG will be designed to provide, a total life cycle cost savings by reducing both operational and maintenance costs when compared to the MK-7 arresting gear.  The AAG will also provide new operational capabilities, including the ability to safely and efficiently recover both heavier and/or faster aircraft and lightweight unmanned air vehicles (goal) that may enter the Fleet in the coming years.

2.  Physical Description.  The AAG Basic System for an aircraft carrier is composed of four AAG engines (three pendant engines and one barricade engine), and one basic operational control system.  The AAG Basic System will also include the operator workstations and external interfaces that the AAG requires.  The AAG system will interface with the current MK-7 sheave damper, fairlead, and deck sheave systems, as well as the purchase cable and Cross-Deck Pendants (CDP).  The AAG basic System will be upgradeable to the Pre-Planned Product Improvement (P3I) System with minimal redesign.

The AAG P3I System will be comprised of the following P3I block upgrades, in order of preference:

- Option (1): Enhanced Control System for the Basic AAG system with the following capabilities: conditioned based maintenance, automated diagnostic troubleshooting and prognostic capability, reach-back maintenance capability, Interactive Electronic Technical Manuals, continuous system status condition monitoring, Built-In Test/Built-In Test Equipment (BIT/BITE), embedded operator training, and other enhancements.  The Enhanced Control System will include the capability to provide for an Integrated Arresting Gear Control Station.

- Option (2): Redesigned sheave damper system (may be an active control system) with capabilities compatible with the Enhanced AAG Control System.

- Option (3): New high-strength, lightweight Purchase Cable and CDP.

The AAG system consists of four units, with a unit defined as a single recovery wire and associated equipment.  It is envisioned that the AAG deck configuration will utilize a “3 + 1” recovery wire configuration, where a maximum of three recovery wires are rigged on three of the units at any given time.  The remaining unit may be utilized as a spare, enabling a recovery wire to be rigged in the event one of the other units becomes unavailable.

3.  New Development Introduction.  The AAG will be developed in two blocks.  Block I will meet the requirements for CVN 68 class backfit and CVNX 1.  Block II will meet the requirements for CVNX 2 and follow-on ships.  Block II will build upon Block I.  Commonality between the two blocks will be maintained to the maximum extent.

4.  Significant Interfaces.  It is currently envisioned that the AAG will have interfaces with other current or future shipboard systems.  Some of the systems listed below are still in the design and development phase and may not be installed in fleet ships prior to AAG being designed and fielded.  If these systems are in place, Aviation Data Management And Control System (ADMACS) will be the single interface for AAG for all these systems.  If these systems are not in place on a ship, the required inputs and outputs will be handled within the AAG Control System:

- ADMACS: ADMACS is a shipboard information management system for Aircraft Launch and Recovery Equipment (ALRE) and Air Operations data and information.  The AAG may use ADMACS for sending and receiving data both within the AAG sub-systems (as appropriate) and with outside systems on the ship (for wind information, etc.).

- Digital Wind: The AAG will get wind information from the MORIAH system.  The ADMACS system will be used to get the MORIAH wind data to the AAG.

- IFLOLS: The Improved Fresnel Lens Optical Landing System (IFLOLS) is a replacement for the current Fresnel Lens Optical Landing System (FLOLS) system.  IFLOLS is currently being installed on Fleet carriers.  Interface with IFLOLS is needed to enable an Arresting Gear/IFLOLS Cross Check functionality (which is an AAG requirement).  Also, the AAG may use the IFLOLS flat panel display/control workstation in Pri-Fly as the primary input-display for AAG in Pri-Fly.

- Cross Check System: The current Cross Check System is a manual pushbutton system, with manual inputs from the Pri-Fly Arresting Gear (AG) Operator and from the FLOLS Operator.  When both operators correctly depress a button indicating the AG system and the FLOLS system are both properly set for the next aircraft type to recover, a light will indicate on the Air Boss’s small display indicating both systems are set for the next aircraft.  The current Cross Check System is an open loop system (i.e., it does not get electronic feedback from either the MK-7 AG or the FLOLS.  The ALRE Team is redesigning the current Cross Check System.  This new Redesigned Cross Check System provides a closed loop feedback from the MK-7 AG to verify the Constant Runout (Control) Valve (CROV) setting and verify battery conditions.  It will also provide an automated function that will actually move the CROV setting to the correct weight setting following input from the Pri-Fly AG Operator.  The Redesigned Cross Check System is dependant on the IFLOLS System being installed on the ship since it will use the IFLOLS workstation in Pri-Fly (a flat panel input-display).  The AAG Program will incorporate all the required functions of the new Redesigned Cross Check System into the AAG control system such that a separate Cross Check System will no longer be required.

- Flight Deck Status Light: The AAG control system will automatically interface with the Flight Deck Status Light such that the light can not indicate a clear deck (green light on) if the AAG is not ready to safely recover the next aircraft.

5.  New Features, Configurations, or Material.  The AAG program will also look at other new technology, as cost and technical risks permit, such as the use of new, high-tech materials for the purchase cable, CDP, or other new technologies and materials.  Previous work has shown significant dynamic load reductions can be realized by utilizing high strength, low weight, purchase cables and/or CDPs.  Higher elasticity as well as low inertia tends to reduce the magnitude of the impact generated kink wave.  The primary limiting factor for these, high strength, low weight materials has been life.  The materials must be capable of surviving the harsh and abrasive sea environment.

H.  CONCEPTS

1.  Operational Concept.  A primary goal of the AAG is to allow recovery operations to be executed using significantly fewer sailors in the arresting gear crew.  The AAG Team’s current vision of the proposed operational concept may only require the following operators for normal operations:

- Pri-Fly Recovery Operator - This operator also performs the necessary functions for operation of the IFLOLS.

- Arresting Gear Officer - At the current deck edge position for the Basic AAG System and in the Integrated Arresting Gear Control System (IAGCS) for the AAG P3I Enhanced Control Option.

- AAG Monitor - Below deck for the Basic AAG System, in the IAGCS for the AAG P3I Enhanced Control Option.

- AAG Retract Operator - Required for the AAG Basic System.  For the AAG P3I System, the AAG Monitor will perform this function.

2.  Maintenance Concept.  The AAG system will have an embedded Health Monitoring (HM) system, an embedded performance monitoring system, an embedded BIT/BITE check system, an embedded Interactive Electronic Technical Manual (IETM), and troubleshooting maintenance aids.  It will have a HM and troubleshooting display console located in the Arresting Gear Work Center spaces and in the V-2 Maintenance Control Office spaces.  The AAG system will also have an embedded routine that tracks hits on individual CDPs that must be changed after every 100 aircraft traps on the CDP.  The AAG will also have an embedded system for tracking any other system component that has a limited life (e.g., purchase cable, etc.).

a.  Organizational.  Using Conditional Based Maintenance (CBM) and HM will allow for the monitoring and diagnosis of the Catapults and Arresting Gear.  CBM and HM will instrument critical parameters and use the data obtained to determine the “health” of the systems.  Using these techniques, AAG will be able to determine when maintenance is required rather than the current event or time-driven Preventive Maintenance method.  With CBM, the maintenance actions themselves may not change, but the frequency of the maintenance actions will be reduced.

(1) Preventive Maintenance.  A supportability analyses will be performed to determine which maintenance actions will be required in support of AAG.

(2) Corrective Maintenance.  As Corrective Maintenance requirements are determined, via the supportability analyses, they will be added to updates to this document.

b.  Intermediate.  ALRE will explore a Reach Back Maintenance capability.  This concept will allow data in various formats to be transmitted ashore to the activity that can provide assistance in direct support of diagnosing catapult or arresting gear problems.

c.  Depot.  Depot level and other major maintenance and repair are available through Voyage Repair Teams provided by Naval Aviation Depots, NAWCADLKE, and Naval shipyards, or a commercial contractor.

d.  Interim Maintenance.  The contractor will provide interim maintenance support.

e.  Life Cycle Maintenance Plan.  After the Design Phase is complete, the contractor will draft a life cycle maintenance plan.

3.  Manning Concept.  Manpower requirements for the V-2 Division are based on total workload requirements.  With a daily operating period of 16 hours, Quality Assurance (QA) and Maintenance Support (MS) capabilities must be available 24 hours per day.  The V-2 Division operates with separate work centers for QA, MS, and operation of AAG.  The V-2 Division is manned with:

- Aviation Boatswain’s Mate (Launching and Recovery Equipment) (ABE) for operation and maintenance of AAG

- Electrician’s Mates (EM) to maintain the AAG electrical systems

- Interior Communications Electricians (IC) to maintain the visual landing systems

- Aviation Maintenance Administrationmen (AZ) to perform the administrative, managerial, trend analysis, and clerical tasks of the division

This same mix of personnel, required to support the AAG system, has not been determined.  The AAG will require no operators in the arresting gear engine rooms during recovery operations (threshold requirement).  The only operators required for the AAG Basic System should be a single Primary Flight Control operator, an Arresting Gear Officer, an operator that can monitor all AAG engines at an integrated console, and one operator to perform the retract function.

a.  Estimated Maintenance Man-Hours per Operating Hour.  The Mean Cycles Between Operational Mission Failures (MCBOMF) for the system for AAG must not be less than 16,500 cycles (threshold) and 29,500 cycles (objective).  One cycle equals one arrestment-retract-ready sequence.  A system operational mission failure occurs if and when a failure results in less than two recovery wires (units) being operationally ready for the next cycle, given a Mean Logistics Delay Time (MLDT) of 60 hours.

The MCBOMF for an individual unit must not be less than 1,400 cycles (threshold) and 1,800 cycles (objective).  The MCBOMF is based on a 3 + 1 wire configuration.  The AAG must meet an organizational maintenance level Mean Time To Repair of 1.00 hour (threshold) and less than 1.00 hour (goal).

b.  Proposed Utilization.  AAG is designated as a non-continuous operating system.  The system must, however, be capable of recovery operations at any time in support of carrier based tailhook equipped fixed-wing air operations.

c.  Recommended Qualitative and Quantitative Manpower Requirements.  At this point in the development of the AAG, it is anticipated that there will not be any immediate change to the current manning aboard CVN ships.  There is a possibility that some reduction in manpower may be realized after final system design and if the CBM philosophy is adopted.  Results of additional analysis will be reflected in revisions to this document.

4.  Training Concept.  AAG is currently in the Concept and Technology Development phase of the DAS and as such, no final training concept can be defined.  When the system specification become final, the training concept can be further addressed.  Current plans are to determine the training methodology after contract award.

Formal training courses are currently available for ABE Class A and C schools.  These courses will require revision to include the new electronic sensors.  The current maintenance course for Steam Catapult Electrician will require a major revision.  The proposed electronic sensors are within the scope of EM Navy Enlisted Classification (NEC) 4672 but are currently not taught to the level required to maintain the AAG.  The AZ training should be updated to reflect the electronic sensors and provide procedures for data collection and analysis.

As part of an Advanced Technology Demonstrator, Naval Air Warfare Center Training Systems Division (NAWCTSD), Orlando, Florida, is exploring embedded training as a concept for AAG operator training for use aboard future generation aircraft carriers and other Navy platforms.

a.  Initial Training.  Initial training will be required for OPEVAL personnel so they can successfully perform testing functions.  Navy Instructors will also require initial training so that they can establish organic follow-on training.

| Title | AAG Initial Training |
| --- | --- |
| Description | This introductory course provides AAG system operation and maintenance training for Instructors and key personnel. |
| Location | To Be Determined (TBD) |
| Length | TBD |
| RFT date | TBD |
| TTE/TD | TBD |
| Prerequisites | ABE, EM, or AZ Rating |

b.  Follow-on Training

(1) Operator.  Follow-on training for shipboard operators in the V-2 Division will be integrated into the existing training courses for the ABE rating listed below.  It is estimated that the addition of AAG training data into these courses will not significantly increase course lengths.

| Title .................... | Aviation Boatswain’s Mate Launch and Recovery Equipment Class A1 |
| --- | --- |
| CIN ..................... | C-604-2012 |
| Model Manager ... | Naval Air Technical Training Center (NATTC) Pensacola |
| Description .......... | This course provides training to the ABE, including: - Basic Hydraulics - Packing and Seals - Safety Wire and Torquing Applications - NAVAIR Manuals - Basic Aviation - Hazardous Materials in Aircraft - Flight Deck Clothing - 3M System - PQS - Tool Control Upon completion, the student will be able to assist in the operations and maintenance of the Catapult and Arresting Gear Systems in a shipboard environment under supervision. |
| Location .............. | NATTC Pensacola |
| Length ................. | 39 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to Initial Operational Capability (IOC)) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - Airman Recruit, Airman Apprentice, Airman, or ABE Rating - Paygrades E-1 through E-4 - ASVAB Test Scores: 5/6/7 AR+MC+SI=130, and 8/9/10/11/12/13/14/15/16/17 AR+MC+AS=130 |

| Title .................... | Aircraft Launch and Recovery Equipment C13 Catapult Class C1 |
| --- | --- |
| CIN ..................... | C-604-2014 |
| Model Manager ... | NATTC Detachment, Lakehurst |
| Description .......... | This course provides training to the ABE, including: - Type C MK-13 MOD 0 Catapult Operation - Type C MK-13 MOD 1 Catapult Operation - Type C MK-13 MOD 2 Catapult Operation Upon completion, the student will be able operate Type C MK-13 series catapults aboard CV and CVN type ships under limited supervision. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 44 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | NEC 7004 |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - ABE Rating - Paygrades E-4 through E-9 - C-604-2012, Aviation Boatswain’s Mate Launch and Recovery Equipment Class A1 Or - DoD Personnel from a Shipyard, Test Facility, and/or Engineering Development Center. |

| Title .................... | Aircraft Launch and Recovery Equipment Arresting Gear |
| --- | --- |
| CIN ..................... | C-604-2025 |
| Model Manager ... | Naval Air Maintenance Training Unit (NAMTRAU), North Island |
| Description .......... | This course provides the aircraft launch and recovery technician with sufficient knowledge of the MK-7 Arresting Gear System, including: - Operational Phases - Component Identification - Basic Troubleshooting - Safety Precautions Upon completion, the student will be able to perform arresting gear maintenance in a shipboard environment under close supervision. |
| Location .............. | - NAMTRAU Norfolk - NAMTRAU North Island |
| Length ................. | 9 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - ABE Rating - Paygrades E-1 through E-9 (May be a Non-Designated Airman Striking for ABE Rating) |

| Title .................... | Aircraft Launch and Recovery Equipment Arresting Gear |
| --- | --- |
| CIN ..................... | C-604-2029 |
| Model Manager ... | NATTC DET Lakehurst |
| Description .......... | This course provides training to the ABE, including: - MK-7 MOD 2 Arresting Gear Operation - MK-7 MOD 3 Arresting Gear Operation - MK-7 MOD 4 Arresting Gear Operation Upon completion, the student will be able to operate MK-7 series arresting gear aboard CV and CVN type ships under supervision. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 24 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | NEC 7005 |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - ABE Rating - Paygrade E-4 - C-604-2012, Aviation Boatswain’s Mate Launch and Recovery Equipment Class A1 |

| Title .................... | Aircraft Launch and Recovery Equipment Refresher |
| --- | --- |
| CIN ..................... | C-604-2016 |
| Model Manager ... | NAMTRAU North Island |
| Description .......... | This course provides training to the Personnel Qualification Standards (PQS) qualified ABE, including: - Type C MK-13 Series Catapult Operation Upon completion, the student will be able operate MK-13 series catapults aboard CV and CVN type ships under limited supervision. |
| Location .............. | - NAMTRAU Norfolk - NAMTRAU North Island |
| Length ................. | 11 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - ABE Rating - Paygrades E-5 through E-9 |

(2) Maintenance

| Title .................... | Aircraft Launch and Recovery Equipment - Catapult Basic |
| --- | --- |
| CIN ..................... | C-604-2024 |
| Model Manager ... | NAMTRAU North Island |
| Description .......... | This course provides training to ABE, EM, and AZ personnel, including: - Basic Catapult System - Catapult Operational Phases - Component Identification - Basic Troubleshooting - Operation and Maintenance Publications - Safety Precautions Upon completion, the student will be able to perform basic catapult maintenance functions aboard CV and CVN ships under close supervision. |
| Location .............. | - NAMTRAU Norfolk - NAMTRAU North Island |
| Length ................. | 10 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - ABE Rating - Paygrades E-1 through E-9 (may be a Non-Designated Airman Striking for ABE rating) |

| Title .................... | Aircraft Launch and Recovery Equipment Maintenance Technician |
| --- | --- |
| CIN ..................... | C-604-2028 |
| Model Manager ... | NATTC DET Lakehurst |
| Description .......... | This course provides training to the ABE, including: - ALRE Maintenance Administration - Maintenance Programs and Practices - Safety - General Maintenance and Upkeep - Hydraulic System Maintenance - Jet Blast Deflectors - Aircraft Recovery Equipment - Barricades Upon completion, the student will be able to maintain and repair the catapult and arresting gear aboard CV and CVN ships without supervision. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 88 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | NEC 7006 |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisite ......... | - ABE Rating - NEC 7004 or 7005 - Paygrades E-5 through E-9 |

| Title .................... | CV Catapult Electrician |
| --- | --- |
| CIN ..................... | C-604-2013 |
| Model Manager ... | NATTC DET Lakehurst |
| Description .......... | This course provides training to the EM, including: - Arresting Gear and Deck Accessories - Catapults - Electrical Schematics - General Maintenance and Upkeep - Safety - Quality Assurance - Technical Publications Upon completion, the student will be able to maintain and repair the catapult and arresting gear electrical systems aboard CV and CVN ships under limited supervision.. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 26 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | NEC 4672 |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisite ......... | - EM Rating - Paygrade E-4 - Ultimate Duty Assignment to an Aircraft Carrier |

(3) Officer

| Title .................... | Aircraft Launch and Recovery Equipment Officer |
| --- | --- |
| CIN ..................... | C-2G-2010 |
| Model Manager ... | NATTC DET Lakehurst |
| Description .......... | This course provides training to selected officers including: - Basic Hydraulics - Catapult and Arresting Gear Wire Rope and Fittings - Electrical Devices - Familiarization with Steam Catapults - Arresting Gear and Visual Landing Aids System - Detailed Instruction in ALRE Bulletins - Computation - Performing Aircraft Launching Procedures Using Aircraft 11F12 Training Simulator Upon completion, and attaining PQS qualification, the student will be able to safely perform operations of the catapult and arresting gear equipment during actual flight operations without supervision. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 17 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | NOBC 8614 Catapult and Arresting Gear Officer |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | Officers Selected by the Chief of Naval Operations |

| Title .................... | Aircraft Launch and Recovery Equipment Maintenance Officer |
| --- | --- |
| CIN ..................... | C-604-2011 |
| Model Manager ... | NATTC DET Lakehurst |
| Description .......... | This course provides training to selected CWO/LDOs and selected ABEs including: - Common Duties and Responsibilities - ALRE Maintenance Administrative Matters - Technical Publications Library Organization and Management - Catapult Maintenance - Arresting Gear Maintenance and Visual Landing Aids Maintenance - ALRE Maintenance Management - 3M and Ship Alterations Upon completion the student will be able to manage and supervise the administration and quality assurance of aircraft launch and recovery equipment maintenance ashore and afloat in accordance with the ALREMP. |
| Location .............. | NATTC DET Lakehurst |
| Length ................. | 38 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - Officers Selected by the Chief of Naval Operations or - ABE Rating - Paygrades E-7 through E-9 |

(4) Other

| Title .................... | Aircraft Launch and Recovery Equipment Quality Assurance Administration |
| --- | --- |
| CIN ..................... | C-604-2017 |
| Model Manager ... | NAMTRAU Norfolk |
| Description .......... | This course provides training to the ABE, EM, or Aviation Maintenance Administrationman, including: - ALRE Quality Assurance Program Overview - Quality Assurance Instructions and Directives - Quality Assurance Record Maintenance - Quality Assurance Reports - Monitoring Procedures Upon completion, the student will be able to administer and maintain a Quality Assurance Program aboard CV and CVN ships under all conditions of readiness under limited supervision. |
| Location .............. | - NAMTRAU Norfolk - NAMTRAU North Island |
| Length ................. | 5 days |
| RFT date ............. | Currently available (AAG revisions are required 90 days prior to IOC) |
| Skill identifier ..... | None |
| TTE/TD .............. | Additional TTE and TDs to support AAG are TBD. |
| Prerequisites ........ | - AZ Rating - Paygrades E-4 through E-6 - Assigned to the V-2 Division Or - ABE, IC, EM Ratings - Paygrades E-6 through E-9 |

c.  Student Profiles

| SKILL IDENTIFIER | PREREQUISITE SKILL AND KNOWLEDGE REQUIREMENTS |
| --- | --- |
| ABE | - C-604-2012, Aviation Boatswain’s Mate Aircraft Launching and Recovery Equipment Class A1 |
| EM | - A-662-0159, Electrician’s Mate “A” School - A-651-0118, Engineering Common Core - A-651-0119, Engineering Electrical Core |
| AZ | - C-555-2010, Aviation Maintenance Administrationman Class A1 |

d.  Training Pipelines.  No new training pipelines or tracks have been identified at this early stage in the acquisition process, but the following courses have been identified as requiring revisions:

| CIN | ### COURSE TITLE |
| --- | --- |
| C-2G-2010 | Aircraft Launch and Recovery Equipment Officer |
| C-604-2011 | Aircraft Launch and Recovery Equipment Maintenance Officer |
| C-604-2012 | Aviation Boatswain’s Mate Launching and Recovery Equipment Class A1 |
| C-604-2013 | CV Catapult Electrician |
| C-604-2014 | Aircraft Launch and Recovery Equipment C13 Catapult Class C1 |
| C-604-2016 | Aircraft Launch and Recovery Equipment Refresher |
| C-604-2017 | Aircraft Launch and Recovery Equipment Quality Assurance Administration |
| C-604-2024 | Aircraft Launch and Recovery Equipment – Catapult Basic |
| C-604-2025 | Aircraft Launch and Recovery Equipment – Arresting Gear |
| C-604-2028 | Aircraft Launch and Recovery Equipment Maintenance Technician |
| C-604-2029 | Aircraft Launch and Recovery Equipment – Arresting Gear |

I.  ONBOARD (IN-SERVICE) TRAINING

1.  Proficiency or Other Training Organic to the New Development.  TBD

a.  Maintenance Training Improvement Program.  TBD

b.  Aviation Maintenance Training Continuum System.  TBD

2.  Personnel Qualification Standards.  PQS will be developed by the PQS development center as required.

3.  Other Onboard or In-Service Training Packages.  The curriculum package for both operator and maintenance personnel must be delivered to the user activity and schoolhouses 90 days prior to IOC.  As stated earlier, NAWCTSD is exploring embedded training as a concept for AAG operator training for use aboard future generation aircraft carriers and other Navy platforms.

J.  LOGISTICS SUPPORT

1.  Manufacturer and Contract Numbers.  TBD

2.  Program Documentation

- Operational Requirements Document for AAG, dated 6 September 2001

- System Specifications for AAG dated 30 July 2001

- Program Plan and Plan of Action and Milestones (POA&M) for AAG draft dated 13 December 2000

3.  Technical Data Plan.  Preliminary operator and maintenance manuals will be required, as well as final operation and maintenance manuals with illustrated parts breakdowns, to be prepared by NAWCADLKE.  Manuals will be developed in accordance with a Technical Manual Contract Requirement generated by the Naval Air Technical Data and Engineering Service Command.  The formal technical manuals will be ready for printing and distribution approximately 90 days after receipt of verification of fleet comments.  Commercial manuals will be used where applicable.

4.  Test Sets, Tools, and Test Equipment.  TBD

5.  Repair Parts.  TBD

6.  Human Systems Integration.  The Human Systems Integration program will be organized to achieve the effective integration of personnel into the design of the system.  The human engineering effort will include, but not necessarily be limited to, active participation in the following three major interrelated areas of system development: analysis, design and development, and test and evaluation.  The use of Non-Developmental Item, Commercial Off-The-Shelf (COTS), and Government Off-The-Shelf (GOTS) hardware, software, and firmware common to other systems on the ship should not require new personnel specialties but rather an extension of these skill levels.  Further, the use of highly reliable, integrated, common support systems should result in the more efficient use of operating and support personnel.

### K.  SCHEDULES

1.  Program Phases.  AAG is in the Concept and Technology Development phase of the DAS.  Phase B System Development and Demonstration phase of the DAS is expected to begin in FY08.  Phase C decision is currently scheduled to begin in FY13.

2.  Initial Operational Capability.  IOC must occur when the first ship installed with AAG is deployed in 2016.

3.  Full Operational Capability.  Full Operational Capability will occur when the Navy Support Date for the AAG has been achieved.

4.  Installation and Delivery Schedules.  TBD

5.  Foreign Military Sales and Other Source Delivery Schedule.  Not Applicable (NA)

6.  Training Device and Technical Training Equipment Delivery Schedule.  TBD

L.  GOVERNMENT-FURNISHED EQUIPMENT AND CONTRACTOR-FURNISHED EQUIPMENT TRAINING REQUIREMENTS.  NA

### M.  RELATED NTSPs AND OTHER APPLICABLE DOCUMENTS

| DOCUMENT OR NTSP TITLE | DOCUMENT OR NTSP NUMBER | PDA CODE | STATUS |
| --- | --- | --- | --- |
| Initial NTSP for ALCRS | NA | PMA251 | Initial Oct 99 |
| NTSP for ADMACS | N78-NTSP-A-50-0009/D | PMA251 | Approved Mar 02 |
| NTSP CV/CVN for ALRE | N88-NTSP-A-50-8509D/D | PMA251 PMW185 | Proposed Jul 02 |
| NTSP for MORIAH | N88-NTSP-A-50-0001/A | PMA251 | Proposed Jun 02 |
| Program Plan and Plan of Action and Milestones | NA | PMA251 | Draft Dec 00 |

CAPT Owen Fletcher	COMM:	(703) 604-7747

Deputy Aviation Maintenance Programs	DSN:	664-7747

CNO, N781B	FAX:	(703) 604-6972

fletcher.owen@hq.navy.mil

CDR Wanda Janus	COMM:	(703) 602-7720

Resource Sponsor / Program Sponsor	DSN:	227-7720

CNO, N785D1	FAX:	(703) 695-8523

janus.wanda@hq.navy.mil

CAPT Terry Merritt	COMM:	(703) 604-7730

Head, Aviation Technical Training Branch	DSN:	664-7730

CNO, N789H	FAX:	(703) 604-6939

merritt.terry@hq.navy.mil

AZCS Gary Greenlee	COMM:	(703) 604-7709

NTSP Manager	DSN:	664-7709

CNO, N789H7	FAX:	(703) 604-6939

greenlee.gary@hq.navy.mil

CDR Kevin Neary	COMM:	(703) 695-3247

Aviation Manpower	DSN:	225-3247

CNO, N122C1	FAX:	(703) 614-5308

n122c1@bupers.navy.mil

Mr. Robert Zweibel	COMM:	(703) 602-5151

Training Technology Policy	DSN:	332-5151

CNO, N795K	FAX:	(703) 602-5175

zweibel.robert@hq.navy.mil

AZCM Kevin Green	COMM:	(301) 757-8120

AMTCS Training Systems Manager	DSN:	757-8120

NAVAIRSYSCOM, PMA205-3D3	FAX:	(301) 757-6941

greenkl@navair.navy.mil

CDR Mike Hohl	COMM:	(757) 836-0085

Aviation NTSP Point of Contact	DSN:	836-0085

CINCLANTFLT, N731	FAX: 	(757) 836-6737

hohlmj@clf.navy.mil

CAPT Pat Salsman	COMM:	(757) 863-6495

Branch Head, Training Requirements and Assessments	DSN:	863-6495

CINCLANTFLT, N72	FAX: 	(757) 863-6794

salsmancp@clf.navy.mil

Mr. Bob Long	COMM:	(808) 471-8513

Deputy Director for Training	DSN:	315-471-8513

CINCPACFLT, N70 	FAX: 	(808) 471-8596

longrh@cpf.navy.mil

CAPT Patricia Huiatt	COMM:	(901) 874-3529

Deputy Assistant, Chief of Naval Personnel for Distribution	DSN:	882-3529

NAVPERSCOM, PERS-4B	FAX: 	(901) 874-2606

p4b@persnet.navy.mil

CDR Timothy Ferree	COMM:	(901) 874-3691

Branch Head, Aviation Enlisted Assignments	DSN:	882-3691

NAVPERSCOM, PERS-404	FAX:	(901) 874-2642

p404@persnet.navy.mil

CDR Rose Wynn	COMM:	(901) 874-6218

Aviation Department Head	DSN:	882-6218

NAVMAC, 30	FAX:	(901) 874-6471

rose.wynn@navmac.navy.mil

SKCS Parthina Jacobs	COMM:	(901) 874-6483

NTSP Coordinator (Assistant)	DSN:	882-6483

NAVMAC, 32 	FAX:	(901) 874-6471

parthina.jacobs@navmac.navy.mil

Mr. Dick Bushway	COMM:	(301) 757-7008

Developing Agency	DSN:	757-7008

NAVAIRSYSCOM, PMA251	FAX:	(301) 757-6800

bushwayd@navair.navy.mil

Ms. Sonya Smith	Comm:	(301) 757-7001

Developing Agency 	DSN:	757-7001

NAVAIRSYSCOM, PMA251 	FAX:	(301) 757-6800

Smithsm@navair.navy.mil

AZCM Kevin Green	COMM:	(301) 757-8120

AMTCS Training Systems Manager	DSN:	757-8120

NAVAIRSYSCOM, PMA205-3D3	FAX:	(301) 757-6941

greenkl@navair.navy.mil

Mr. Armando Machado	COMM:	(732) 323-7191

Logistics Management Specialist	DSN:	624-7191

NAWCADLKE AIR 3.4.5	FAX:	(732) 323-4064

Machadoaj@navair.navy.mil

AVCM Thomas King	COMM:	(850) 452-9712 ext. 249

Training Coordinator	DSN:	922-9712 ext. 249

NAMTRAGRU HQ, N2213	FAX:	(850) 452-9965

avcm-thomas.e.king@smtp.cnet.navy.mil

CAPT Grant Ziebell	COMM:	(850) 452-4330

CNET NTSP Coordination	DSN:	922-4330

CNET ETS3	FAX:	(850) 452-4853

capt-grant.ziebell@cnet.navy.mil

CDR Erich Blunt	COMM:	(850) 452-4915

Aviation Technical Training	DSN:	922-4915

CNET, ETE-32	FAX:	(850) 452-4901

cdr_erich.blunt@smtp.cnet.navy.mil

AVCM Robert Clare	COMM:	(850) 452-1001 ext. 2246

PQS Development Group LCPO	DSN:	922-1001 ext. 2246

NETPDTC	FAX:	(850) 452-1764

avcm-robert.clare@smtp.cnet.navy.mil

Mr. Phil Szczyglowski	COMM:	(301) 757-8280

Competency Manager	DSN:	757-8280

NAVAIRSYSCOM, AIR 3.4.1	FAX:	(301) 342-7737

szczyglowspr@navair.navy.mil

Mr. Bob Kresge	COMM:	(301) 757-1844

NTSP Manager	DSN:	757-1844

NAVAIRSYSCOM, AIR 3.4.1	FAX:	(301) 342-7737

kresgerj@navair.navy.mil

Mr. Gary Barnes	COMM:	(301) 757-8289

Manpower and Training Analyst (NTSP Author)	DSN:	757-8289

NAVAIRSYSCOM, AIR 3.4.1	FAX:	(301) 342-7737

barnesgd@navair.navy.mil